10.5.	自然言語理解 / Natural Language Understanding

10.5.1. On-board NLU　▲c

所謂Wide Grammar方式ではない、NLUソリューションを採用。対象タスクは、6.6章　機能一覧を参照。

Target task shall be referred section 6.6. Function list.

Below descriptions are the specific requirements for NLU component.

Multi-Language Correction / ASR Result Word Correction▲j

Based on the text provided to the NLU, corrections can be made and published. Also, Individual word by word correction based on pre-existing context or reference lists. (i.e. ASR result= " Tacos e mas" vs NLU correction = "Tacos Y Más")

End-User Preference Mapping / Contextual Understanding

Context from the end-user profile used in determining the intent. Also, the on-board NLU shall receive and process inputs from other components/modules to understand overall context. For example: (1) vehicle inputs such as speed, (2) outside condition such as weather, (3) inside vehicle condition such as drive mode setting, can be received and process to better understand context and provide best possible action/response to user.

Other inputs: user preference (for example: user interest, personal contact, user profile, etc. provided with respect to privacy)

(i.e. user prefer's shell gas station. When user request for "gas station", system to provide selection of Shell station.)

Multi-criteria and -intent support　▲d

The on-board NLU shall be able to handle utterances that contain multiple criteria (for example: phone type, payment type, structure type, food type, etc.) and/or intent that involves single or multiple domain. For example: Multiple Domains –

USER: “Find a covered parking nearby destination that accepts credit card and turn on the AC”

Domain1: Navigation

Domain2: Vehicle Control

Intent1: “Find <POI> near destination”

Intent2: “Turn AC On”

Criteria1: “Covered parking”

Criteria2: “Accepts credit card”

Conversational Support▲j

The on-board NLU shall be able to resolve ambiguity by the help of the end-user to complete a task. For example:

USER: “How’s the weather like in Pensacola?”

VA: “In Florida?”

USER: “In Oklahoma”

Smart Interaction▲j

The on-board NLU shall be able to fill in missing information and remove ambiguous references from the context. For example:

USER: “What’s the weather like today?”

VA: “Weather today is sunny with a HI of 95 and a LO of 82”

**user starts a new session subsequently**

USER: “What about in Pensacola?”

VA: “Weather in Pensacola, Oklahoma is partly cloudy with 30% change of rain”

System understands the context based on the previous command/session context (i.e. weather context) and assumes “Pensacola in Oklahoma” based on command history.

Partial Recognition▲j

The on-board NLU’s ability to handle incomplete commands. The on-board NLU shall be able to understand the intent of a command and provide actions even when command is missing key information. For example:

USER: “Can you please go to 8777 Platt Road”

VA: “Navigating to 8777 Platt Road Saline, Michigan”

Real-time Processing

The on-board NLU shall be capable of processing individual words and/or segments as it is received (in real-time).

Follow-up questions▲j

The NLU should keep references to entities from requests, which can then be used to answer follow up commands.

Example: “Is Joe’s Pizza still open?” “Joe’s Pizza will close in 45 minutes” “Ok, navigate there”

Follow-up questions should be supported, even if a limited amount of other commands were being issued in the meanwhile.

10.5.2. Off-board NLU　▲aab

Hybrid音声認識の対応仕向けにおいて、サーバとの通信が可能な状態においては、Off-boardでも自然言語理解に対応する。

In case off-board voice recognition is supported, Off-board NLU shall be used as far as data connection is available.